Mitchenkov Dmitriy

Problem 1. Compute (derive math formulas) the Fourier transform

a. fx=x∙e-ax

fx=x∙e-ax

Fx∙e-axω=i∙ddωFe-axω

i∙Fe-axω'=i∙12∙π∙ddω2∙aa2+ω2=-4∙a∙ω∙i2∙πω2+a22

ddω2∙aa2+ω2=2∙a'∙a2+ω2-2∙a∙a2+ω2'a2+ω22=-4aωa2+ω22

Answer:

-4∙a∙ω∙i2∙πω2+a22

b. fx=e-a2x2cosb∙x

cosb∙x=eix+e-ix2 (euler formula)

fx=e-a2x2∙eibx+e-ibx2=12e-a2x2∙eibx+e-a2x2∙e-ibx

Ff1x+f2xω=Ff1xω+Ff1xω

Ffxω=Fe-a2x2cosb∙xω=F12e-a2x2∙eibx+e-a2x2∙e-ibxω=

=12∙Fe-a2x2∙eibxω+Fe-a2x2∙e-ibxω

So we need to find Fourier transform from 2 signals: f1=e-a2x2∙eibx; f2=e-a2x2∙e-ibx

- Fe-a2x2∙eibxω

Fe-a2x2∙eibxω=Fe-a2x2ω-b

Replace the variables: a2=12σ2; σ=12a2=12∙a

Fe-a2x2ω-b=Fe-x22σ2ω-b=σ∙2∙π∙e-σ2∙ω-b22=

=12π12∙a∙2∙π∙exp-12∙a2∙ω-b22=12a∙exp-12a2∙ω-b22=

12aexp-ω-b24a2=12aexp-ω-b2a2

- The same for Fe-a2x2∙ei∙-b∙xω:

Fe-a2x2∙ei∙-b∙xω=Fe-a2x2ω+b)=12aexp-ω+b2a2

Final result:

1212aexp-ω+b2a2+12aexp-ω-b2a2=

=18aexp-ω+b2a2+exp-ω-b2a2

Answer:

18aexp-ω+b2a2+exp-ω-b2a2

Problem 2. Compute (derive math formulas) the spectrum of the sine-filled sinc function

fx=sinc(a∙x)∙sinb∙x

cosb∙x=eix-e-ix2i (euler formula)

Fsinc(a∙x)∙sinb∙xω=2i∙(Fsinca∙xω- b-Fsinca∙xω+ b)

F1π∙πsinca∙xω=π∙β(a∙ω)

Fsinca∙x∙sinb∙xω=2i2ππ∙βa∙ω- b-π∙βa∙ω+ b=

i2πβa∙ω- b-βa∙ω+ b

βt=rectangular pulse

Problem 3. Compute (derive math formulas) the autocorrelation of the triangular pulse

- Compute (derive math formulas) the autocorrelation of the triangular pulse

| 1 |  | t-1>1=>t>2 y1t=0 |
| --- | --- | --- |
| 2 |  | 0<t-1<1=>1<t<2 y2t=t-111-τ1+τ-tdτ |
| 3 | -1<t-1<0=>0<t<1 y3t=t-101+τ1+τ-tdt+0t1-τ1+τ-tdτ+t11-τ1-τ+tdτ =16t3-3t+2+16tt2-6t+6+16t3-3t+2=13t3-3t+2+16tt2-6t+6 |  |
| 4 | 0<t+1<1=>-1<t<0 y4t=-1t1+τ1+τ-tdt+t01+τ1-τ+tdτ+0t+11-τ1-τ+tdτ=16-t3+3t+2-16tt2+6t+6+16-t3+3t+2=13-t3+3t+2-16tt2+6t+6 |  |
| 5 |  | -1<t+1<0=>-2<t<-1 y5t=-1t+11+τ1-τ+tdτ=16(t+2)3 |
| 6 |  | t+1<-1=>t<-2 y6t=0 |

b)

Problem 4. Compute (by hands) the convolution of the following signals:
a. h = [2 3 6 8]; x = [1 2 10 1]; b. h = [5 1 3 10]; x = [9 6 10 1];

Step 1.

|  |  | x |  |  |  |  |  |  | x |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 10 | 1 |  |  |  | 9 | 6 | 10 | 1 |
| h | 2 | 2 | 4 | 20 | 2 |  | h | 5 | 45 | 30 | 50 | 5 |
|  | 3 | 3 | 6 | 30 | 3 |  |  | 1 | 9 | 6 | 10 | 1 |
|  | 6 | 6 | 12 | 60 | 6 |  |  | 3 | 27 | 18 | 30 | 3 |
|  | 8 | 8 | 16 | 80 | 8 | ←hi∙xj |  | 10 | 90 | 60 | 100 | 10 |

Step 2.

|  |  | x |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 10 | 1 |  |  |  |  |  |  |  | 9 | 6 | 10 | 1 |  |  |  |
| h | 2 | 2 | 4 | 20 | 2 | 0 | 0 | 0 |  |  | h | 5 | 45 | 30 | 50 | 5 | 0 | 0 | 0 |
|  | 3 | 0 | 3 | 6 | 30 | 3 | 0 | 0 |  |  |  | 1 | 0 | 9 | 6 | 10 | 1 | 0 | 0 |
|  | 6 | 0 | 0 | 6 | 12 | 60 | 6 | 0 |  |  |  | 3 | 0 | 0 | 27 | 18 | 30 | 3 | 0 |
|  | 8 | 0 | 0 | 0 | 8 | 16 | 80 | 8 |  |  |  | 10 | 0 | 0 | 0 | 90 | 60 | 100 | 10 |
|  |  | 2 | 7 | 32 | 52 | 79 | 86 | 8 |  |  |  |  | 45 | 39 | 83 | 123 | 91 | 103 | 10 |

Step 3.

Problem 5. The sinusoidal signal with the frequency 6kHz ….
a. … is sampled with the frequency 10 kHz. Compute the apparent frequency after the signal reconstruction.

The signals in time domain

The signals in frequency domain

Answer: The reconstructed with Nyquist filter signal will have the frequency equal to 4kHz.

b. … is represented by 4 signal periods sampled with the sample clock frequency 15 kHz. Are these samples enough to reconstruct the initial signal correctly (without ANY error)? Plot the original and the reconstructed signals. Explain the difference.

The reconstruction of signal using Nyquist filter

The reconstructed signal

Answer: this quantity of samples is not enough to reconstruct the limited in time signal. With incrasing a number of periods error will decrease

MATLAB code for task №5:

close all

clear all

fss = 15e3;  % sampling frequency for modeling

fs = 15;     % sampling frequency

f_signal = 6;  % signal frequency

w =2*pi*f_signal;  % signal angular frequency

Num_of_periods = 4;  % number of periods of signal

t = Num_of_periods/f_signal;  % modeling time: 0..t

time_signal = 0:(1/fss):t;  % time counts for modeling

time_sampled = 0:(1/fs):t;  % time counts for discretization

signal = sin(w*time_signal);  % raw signal

discrete = sin(w*time_sampled);  % sample counts

figure(1);plot(time_signal,signal);

hold on;

% plot(time_sampled,discrete,'ks')

stem(time_sampled,discrete,'ks--');

% попытка восстановить сигнал с использованием синков

tt = (-t:(1/fss):t);  % time counts for filter (based on modeling frequency)

Nyquist_filter = sinc(fs*tt);  % Nyquist filter in time domain

% plot(tt,Nyquist_filter);

upsampled = upsample(discrete,fss/fs);  % upsample discrete signal to the modeling frequency (new samples is equal to zero)

upsampled(end+1-(fss/fs-1):end) = [];  % remove last unused zeros: S000S000S[000] (S = sample)

restored = conv(upsampled,Nyquist_filter,'same');  % restored signal: signal -> sample -> filter

ttt = linspace(0,t,length(restored));

figure(2);hold on;

plot(ttt,restored);stem(time_sampled,discrete);plot(time_signal,signal);

legend('restored','sampled','original');

figure(4);hold on;

fft_restored = abs(fft(restored)/length(restored));

plot(linspace(0,fss,length(restored)),fft_restored)

fft_sampled = abs(fft(upsampled)/length(upsampled)).*fss/fs; %energy loss compensation

plot(linspace(0,fss,length(upsampled)),fft_sampled)

fft_signal = abs(fft(signal)/length(signal));

plot(linspace(0,fss,length(signal)),fft_signal)

legend('restored','sampled','original');xlim([0, 2*fs]);set(gca, 'YScale', 'log')

% return

[rId, cId] = find(upsampled);

figure(3);hold on;

plot(ttt,restored);stem(time_sampled,discrete);

for i = cId

plot(tt+((i-1)*1/fss),upsampled(i)*Nyquist_filter,'b--');

end

xlim([0 t]);